BOGDAN STĂNESE – ILIE

Tracker Project

An application

CONTENTS

- Introduction

  - General description

  - A practical idea for fulfilling the need of a tracking device

  - Aspects of improvement

- Theoretical backround

- Implementation

  - Hardware

    - Electrical schematic

    - PCB & ECAD

    - BOM

    - Chasis

  - Software

    - Programming language & IDE

    - Interpreter

    - Program

- Conclusions

- Bibliography

  - Source code - Interpreter

  - Documentation

  - Examples on “in action” aproaches

Introduction

General description

Tracker device practically has a GPS module which keeps track of location by connecting itself to satellites around the globe and it returns data in a format similar to a vector, each field being separated by comma, and a GSM module, it receives the parsed data from GPS and by a simple algorithm of decisions, it selects each user who would try to access the device by sending a certain SMS to GSM module, the user can get approximately where the device is.

A practical idea

In some situations everyone wants to know where an important object is, for example, their phone, keys, wallet, laptop, bike, tablet, satchel, bag, purse, car, motorbike. This approach tries to fit in, but not for such small objects, like keys. It can be used inside cars or bags.

Aspects of improvement

The device size can be reduced to be used with many other gadgets. GSP and GSM modules are on separate, but much smaller, PCBs, an idea would have been to have each IC on the main board to decrease total size of device. Power supply part had not been implemented on PCB before the overall project to be done, which later would have come in handy to have all things in one place, and from this drawback, tracker device needs an external supply to power it through USB-B. Program efficiency can be increased by adding layers of error checking.

Theoretical backround

The project came with idea to have basically a working program which would have combined GPS module with a simple OLED display by using a development board. With the help of such boards we can create robust programs for prototyping, controlling and automating machine insides factories. Later on, all in one place circuits from a development board can bring up powerfull features, for example HDMI, audio codec, MIPI interfaces. This lead to the idea of having a personal and trackable device which brought in discussion the GSM module. It integrates with GSP seamlessly through a microcontroller from Raspberry Py, RP2040 which offers multi-threading features, offering a big bonus for the overall program.

Implementation

Hardware

Electrical Schematic

Power supply

An LDO voltage regulator is used to decrease the voltage from 5V to 3V3 because of GPS module. LD1117 has through-hole pins to make it easier for soldering. Decoupling capacitors help in reducing ΔVripple.

LEDs

Leds were used to check on runtime of program what it does more precisely. RP2040 generates sufficient current to drive LEDs.

Pi Pico board

Decoupling capacitors are used for Vsys pin of Pi Pico board to maintain ΔVripple as low as possible. The board has a switching regulator to obtain 3V3 from 5V to supply RP2040 microcontroller.

GPS module

GPS IC is mounted on a separate PCB and its functionalities can be accessed through an external header placed on its PCB.

GSM module

GSM IC as GPS IC is on its own board, it is powerd up by a switching regulator. It responses at AT commands

PCB & ECAD

In project development KiCad 6.0 and Proteus tools were used, but Proteus being a more powerful ECAD tool the final PCB design is implemented on the physical board.

Top part of PCB has almost all components, but there are on bottom part as well to route easily traces. D1 is the power LED, it take 3V3 from output of regulator to indicate the board is powerd up. Vcc of GPS module is routed directly on top to output of regulator. The USB Vcc pin is routed on bottom of PCB, regulator takes 5V from ‘+’ pin of C12, electrolytic capacitor. Under GSM module two Vias connect Vcc of USB to power up Pi Pico board at VSYS pin.

Bottom part of PCB has traces routed for D2 and D3 to GPIO 16, GPIO17 respectively. GPS module’s Tx and Rx are routed to GPIO5 and GPIO4, GSM module’s Tx and Rx are routed to GPIO1 and GPIO0, these are for the default UART from RP2040. The actual ground planes are created with solder masks for the GND pins.

BOM

| Component Count: | 19 |  |  |
| --- | --- | --- | --- |
| Ref | Qnty | Value | Cmp name |
| U2 | 1 | SC0915 | SC0915 |
| C3, C4, C7 | 3 | 100nF | Tantalum type-D |
| C12, C1, C2, C6, C5 | 5 | 100uF | Tantalum type-D |
| D1 | 1 | LED_power | LED |
| D2 | 1 | LED_gps | LED |
| D3 | 1 | LED_gsm | LED |
| USB | 1 | USB_B | USB_B |
| R1 | 1 | 300Ω | 1206 |
| R2, R3 | 2 | 300Ω | 1206 |
| U3 | 1 | LD1117_TO-220 | LD1117_TO-220 |
| U2 | 1 | Pmod_GPS | Pmod_GPS |
| U1 | 1 | GSM_module | GSM_module |

Chasis

Software

Programming language & IDE

Python is used for the whole program, files with source code are placed on an external flash memory from Pi Pico board and inside the RP2040 microcontroller runs MicroPython interpreter. It is placed by pressing the bootloader button from board before connecting it to an laptop through USB port and realeasing it afterwards. RP2040 will be seen as a Mass Storage Device in which by simply ‘Drag & Drop’, the interpreter can be inserted. PyCharm IDE has MicroPython Plugin, but it does not work perfectly. Thonny IDE was developed for Raspberry Py microcontrollers which works better. It is used to place source code files in the flash memory.

Interpreter

MicroPython has a good documentation and online source code on GitHub. It can be used on various microcontrollers, for example esp8266, stm 32I0, nRF52840, mimxrt, cc3200, RA4M1.

Program

All files are linked through main.py file, gps_linker.py has a class specific for GPS module with methods which parse data coming from IC, gsm_linker.py has a class intended for GSM module with methods for initialization, sending messages/receiving messages. The file reset_if_blocked.py uses WatchDog timer from RP2040 if the program has a chance to block. A class for URL formats is located in url_format.py.

In main function UART module are instantiated with specific baudrate for gsm and gps. Objects for GPS class and GSM class are then created, afterwards GSM is initialized with predefined settings. By using uasyncio module we can create tasks which run independently, await keyword is used to transform each function with async identifier into a generator which is iterated continuously by placing an endless loop at the end of main().

In DataGPS class, contructor __init__() creates initialize attributes which are then used in internal methods. White_space is used to check for spaces when a string of data from gps is received to see if the gps has connected to satellites, if not it will put in string spaces where it did not placed real information. In message will be what GPS transmits on Tx and then parsed by parse_message method. Variables with pos extension represents where the specific items are place in string. Flg_data_is_ready will be true if data is ready to be segmented. Serial_reader_gps is an object to SteamReader class which lets us to access UART.

GSM class has objects for reading and writing to UART via StreamWriter and StreamReader class from uasyncio module. Number will have thelephone number extracted from GSM, msg_from will get what the user sent, index keeps the index of message the GSM sets it to. Max_msg_in_gsm_mem from initialization of GSM, all messages are store in its internal memory, not on SIM card, which are maximum ten. Take_first_msg flag is used to take the index just for first message from GSM when it receives the AT command for sending on Tx all its messages. No_messages flag signals if the GSM has not a single message.

Attributes in URL class are static because they are not in __init__() function, flg_url_state signals if the url is ready.

Wdt object connects to WatchDog timer to reset RP2040 if it blocks.

Conclusions

The project suffered various changes until it was finished, for example, because of lack of documenation for GSM module, it took some time to figure out the Tx and Rx pins work at 3V3 and 5V, and the development of program took more than expected. It is interesting that the whole program is saved by WhatchDog timer, it is not yet very clear if GSM blocks UART communication on Pi Pico board or if some signal on board disrupts the communication, when GSM receives a message or it sends one, which will lead the program to block, because source code files are not in RP2040, but in an external flash memory which RP2040 communicates through QSPI with, and the microcontroller cannot access anymore the files. SIM800L module is a better version of GSM IC.

Bibliography

Source code – Interpreter

- MicroPython - Python for microcontrollers

- micropython/micropython: MicroPython - a lean and efficient Python implementation for microcontrollers and constrained systems (github.com)

Documentation

- Overview — MicroPython latest documentation

- PCBDesignTutorialRevA (alternatezone.com)

- Explore SnapEDA's Symbol & Footprint Libraries | SnapEDA

- Basic AT Commands - ESP32 - — ESP-AT User Guide latest documentation (espressif.com)

Examples on “in action” aproaches

- https://github.com/peterhinch/micropython-async/blob/master/v3/docs/TUTORIAL.md#63-using-the-stream-mechanism

- Uaysncio v3 Stream Writer Drain Bug · Issue #6621 · micropython/micropython (github.com)

- micropython-async/auart_hd.py at master · peterhinch/micropython-async (github.com)